Battery Management System CAN Communication Protocol

Property of Cell Propulsion Private Limited.

223B, KIADB, Bommasandra Industrial Area

Bommasandra, Bengaluru

Karnataka – 560099

www.cellpropulsion.com

1. BMS

BMS monitors the various parameters of the battery such as voltage, temperature, current and keep the battery pack in the safe operating area. BMS communicates with the VCU through controller area network (CAN).

2. Network structure and hardware specification

2.1. CAN network adopts bus topology, the nodes on the network include: Battery Management System and VCU two nodes, each node is connected to the bus through the shielded twisted pair line.

2.2. CAN information frame format standard: 2.0B 29bit Identifier extended frame ID, 120Ω Matching resistance.

2.3. By default, this protocol defaults with termination resistors.

3. Communication conventions of CAN network

CAN network communication rate is 250Kbps

4.  VCU MESSAGE PROTOCOL

VCU sending — Battery management system receiving

| Out | IN | CAN ID | Cycle | Data Length |
| --- | --- | --- | --- | --- |
| VCU | BMS | 0x18F0E5F5 | 100ms | 1 |
| DATA |  |  |  |  |
| Location | Data Name | Description |  |  |
| Byte 0 | BMS state | 0x04 :- Sleep state 0x02 :- Wake Up state 0x05 :- Discharge state 0x06 : - Charge state |  |  |

5.BMS MESSAGE PROTOCOL

Battery management system sending—VCU receiving

| Out | IN | CAN ID | Cycle | Data Length |
| --- | --- | --- | --- | --- |
| BMS | VCU | 0x18F0E5F6 | -- | 8 |
| DATA |  |  |  |  |
| Location | Data Name | Description |  |  |
| Byte0 | BMS current state | 0x04 :- Sleep state 0x02 :- Wake Up state 0x05 :- Discharge state 0x06 : - Charge state |  |  |
| Byte1 | Pack voltage high byte | offset: 0 Conversion: High Byte : 0x22 Low Byte: 0x7A the corresponding voltage in decimal= 8826 is 88.26V |  |  |
| Byte2 | Pack voltage low byte |  |  |  |
| Byte3 | Pack current high byte | offset: 7000 Conversion: High Byte : 0x1B Low Byte: 0x5A the corresponding current in decimal=7002 is (7002-7000) =2A |  |  |
| Byte4 | Pack current low byte |  |  |  |
| Byte5 | Highest Pack Temperature | offset: 40 Conversion: Byte5 : 0x44 the corresponding Temp in decimal=68 is (68-40)=28°C |  |  |
| Byte6 | SOC | offset: 0 Conversion: Byte 6: 0x50 the corresponding soc in decimal=80 is 80% |  |  |
| Byte7 | Fault status | Bit0： Under voltage 0: normal, 1: fault Bit1： Over-voltage 0: normal, 1: fault Bit2： Over temperature 0: normal, 1: fault Bit3： Under temperature 0: normal, 1: fault Bit4： Reserve Bit5： Over-current charge 0: normal, 1: fault Bit6： Over-current discharge 0: normal, 1: fault Bit7： Reserve |  |  |

6. Way of working:

- VCU fixed interval time 100ms send BMS state information to the BMS, the BMS receives the information and according to the message BMS go to the corresponding state.

After powering up, BMS will be in the sleep state. To go to the discharge state, first, send the wake-up state command and then send the discharge state command. Similarly, for the charge state, first send wake up-state command and then send the charge state command. If BMS is not receiving the message for 5 seconds, it enters into the sleep state and turns off the contactor.

2. The BMS sends a broadcast message indicating that the battery status.

7. Flow chart